DESIGN AND IMPLEMENTATION OF AUTOMATED MEDICAL DISPENSER USING ESP32

Project Report

Submitted in partial fulfillment of the requirements for the

Course Title: EMBEDDED PROTOTYPE

Course Code: 23SDEC14R

Submitted by

Devineni Prem Nag

D. Shanmukha Venkata Sai

M. Hemanth Kumar

SK. Sajeeth Shareef

Guided by

Dr. A V. Prabhu

Associate Professor, Dept of ECE

KL UNIVERSITY

Green Fields, Vaddeswaram - 522503, Guntur Dt., AP, India.

Department of Electronics & Communication Engineering

Academic Year: 2024 - 2025

DECLARATION

The Project Report entitled "Design and Implementation of Automated Medical Dispenser using ESP32" is a record of Bonafide work carried out by the students mentioned below, submitted in partial fulfillment for the subject titled 23SDEC14R – Embedded System Application & IoT Programming in Dept of ECE, KL University.

We hereby declare that this project is our original work and that no part of this report has been copied from any other source, University, or Institute, except for proper citations and references.

We also declare that the results embodied in this report have not been submitted to any other examination board or institution for the award of any degree or diploma.

Student Names:   Devineni Prem Nag / D. Shanmukha Venkata Sai / M. Hemanth Kumar / SK. Sajeeth Shareef

Student IDs:      ______________________________

Date:              ______________________________

Signatures:       ______________________________

CERTIFICATION

This is to certify that the Project Report entitled "Design and Implementation of Automated Medical Dispenser using ESP32" is being submitted in partial fulfillment for the subject titled 23SDEC14R – Embedded System Application & IoT Programming in Dept of ECE, KL University. This is a record of Bonafide work carried out under our guidance and supervision.

The results embodied in this report have not been copied from any other department, University, or Institute. The project demonstrates original design, implementation, and hardware work using ESP32 microcontroller, RTC module, servo motor, and IoT connectivity.

| Signature of Examiner | Signature of Supervisor |
| --- | --- |

ACKNOWLEDGEMENT

It is a great pleasure for us to express our sincere gratitude to our honorable President Sri. Koneru Satyanarayana, for providing the opportunity and platform with excellent facilities in accomplishing this project-based laboratory report on Design and Implementation of Automated Medical Dispenser using ESP32.

We record it as our privilege to deeply thank our HOD – ECE for providing us with efficient faculty, state-of-the-art laboratory facilities, and a conducive environment to explore and realize our technical ideas.

We are deeply indebted to our project supervisor, Dr. A V. Prabhu, Associate Professor, Dept of ECE, for their invaluable guidance, continuous motivation, and constructive suggestions throughout this project.

We would also like to thank all the faculty members of the Department of Electronics and Communication Engineering, KL University, for their support and encouragement during the course of this project.

Finally, we are grateful to our families and friends for their unwavering support and encouragement throughout this academic journey.

Names: ______________________________

ID Nos: ______________________________

TABLE OF CONTENTS

| Abstract | 6 |
| --- | --- |
| Chapter 1: Introduction | 7 |
| 1.1 Background and Motivation | 7 |
| 1.2 Problem Statement | 8 |
| 1.3 Scope of the Project | 8 |
| Chapter 2: Project Description | 9 |
| 2.1 System Overview | 9 |
| 2.2 Key Components | 9 |
| Chapter 3: System Requirements | 10 |
| 3.1 Hardware Requirements | 10 |
| 3.2 Software Requirements | 11 |
| Chapter 4: System Design and Architecture | 12 |
| 4.1 Block Diagram | 12 |
| 4.2 Circuit Diagram | 13 |
| 4.3 Flowchart | 14 |
| Chapter 5: Objectives | 15 |
| Chapter 6: Advantages | 16 |
| Chapter 7: Applications | 17 |
| Chapter 8: Results | 18 |
| Chapter 9: Future Scope | 19 |
| Chapter 10: Conclusion | 20 |
| Chapter 11: Project Outcomes | 21 |
| Chapter 12: Team Members | 22 |
| References | 23 |

ABSTRACT

This project presents the design, implementation, and testing of an Automated Medical Dispenser using the ESP32 microcontroller. The primary objective is to address the critical issue of medication non-adherence, especially among elderly patients, by automating the process of medicine dispensing at scheduled times.

The system leverages the ESP32 microcontroller as the central processing unit, combined with an RTC (Real-Time Clock) module for precise time-based scheduling. A servo motor mechanism rotates a pill container to dispense the correct medication at the right time. An LCD/OLED display shows real-time information including the current time and dosage details. A buzzer provides an audible alert to notify the patient when medication is ready.

The IoT connectivity via ESP32's built-in WiFi enables remote monitoring and real-time notifications to caregivers through platforms such as Blynk or Firebase. This system provides a cost-effective, portable, and reliable healthcare solution suitable for home use, elderly care facilities, and hospital environments.

CHAPTER 1: INTRODUCTION

1.1 Background and Motivation

Medication adherence is one of the most critical factors in effective treatment and long-term patient health outcomes. Patients who fail to take their medications on time or at the correct dosage often experience disease progression, unnecessary hospitalizations, and increased healthcare costs. This problem is especially prevalent among elderly individuals, those managing multiple chronic conditions, and patients with cognitive impairments such as dementia.

Traditional pill organizers and manual reminders (such as family members or caregivers prompting patients) are unreliable and not scalable. The growing Internet of Things (IoT) ecosystem offers an opportunity to develop intelligent, automated solutions for medication management that can operate autonomously and communicate with remote caregivers in real time.

1.2 Problem Statement

Many patients, especially elderly people, forget to take medicines on time, leading to serious health complications. Incorrect dosage intake and missed doses are common issues that can have life-threatening consequences for patients with conditions such as diabetes, hypertension, and cardiac diseases. There is a clear need for an automated, reliable, and cost-effective system to ensure patients receive the correct medication at the correct time, with appropriate reminders and remote monitoring capability.

1.3 Scope of the Project

This project focuses on the design and implementation of a hardware-software integrated automated medical dispenser with the following scope:

- Hardware design including ESP32, RTC module, servo motor, motor driver, LCD/OLED display, and buzzer.

- Firmware development for time-based scheduling and motor control.

- IoT integration for remote monitoring and caregiver notifications.

- Testing and validation in a controlled environment.

- The system is designed for home use and small healthcare facilities; it is not intended as a certified medical device.

CHAPTER 2: PROJECT DESCRIPTION

2.1 System Overview

The Automated Medical Dispenser using ESP32 is designed to automatically dispense medicines at pre-scheduled times, reducing the reliance on human memory or manual intervention. The system uses an ESP32 microcontroller as the main control unit, which coordinates all subsystems including time monitoring, motor actuation, display output, audio alerts, and wireless communication.

The RTC module (DS3231) provides highly accurate real-time clock data, ensuring medication schedules are maintained even after power interruptions. When the scheduled time is reached, the ESP32 triggers the motor driver to activate the servo motor, which rotates the pill container to dispense the correct medication into the output tray. Simultaneously, the buzzer sounds an alert and the LCD/OLED display shows the medication information.

2.2 Key Components

| ESP32 Microcontroller | Central processing unit with WiFi & Bluetooth |
| --- | --- |
| RTC Module (DS3231) | Provides accurate real-time clock functionality |
| Servo Motor | Rotates pill container for dispensing mechanism |
| Motor Driver (L298N) | Controls motor direction and speed |
| LCD/OLED Display | Shows current time, schedule, and dosage info |
| Buzzer | Audible alert when medicine is dispensed |
| Push Buttons | Manual override and schedule setting interface |
| IoT Platform | Blynk / Firebase for remote monitoring |

CHAPTER 3: SYSTEM REQUIREMENTS

3.1 Hardware Requirements

| Component | Specification / Purpose |
| --- | --- |
| ESP32 Microcontroller | Main control unit with WiFi/Bluetooth |
| RTC Module (DS3231) | Accurate real-time clock for scheduling |
| Servo Motor / DC Motor | Pill container rotation mechanism |
| Motor Driver (L298N / ULN2003) | Motor control and protection |
| LCD / OLED Display | Time and dosage information display |
| Buzzer | Patient alert notification |
| Push Buttons | Schedule setting and manual control |
| Pill Container | Multi-compartment medication storage |
| Connecting Wires | Circuit connections |
| Breadboard / PCB | Prototyping and circuit mounting |
| Power Supply (5V / 12V Adapter) | System power |

3.2 Software Requirements

| Tool / Platform | Purpose |
| --- | --- |
| Arduino IDE / ESP-IDF | Firmware development environment |
| Embedded C / Arduino Programming | Programming language |
| ESP32 Board Support Package | ESP32 hardware libraries |
| RTClib | RTC module interface library |
| Servo / Motor Control Library | Motor actuation control |
| ESP32 WiFi Library | Wireless connectivity |
| IoT Platform (Blynk / Firebase / ThingSpeak) | Remote monitoring dashboard |
| Serial Monitor | Debugging and testing |

CHAPTER 4: SYSTEM DESIGN AND ARCHITECTURE

4.1 Block Diagram

The following block diagram illustrates the high-level architecture of the Automated Medical Dispenser system, showing the interconnection between all major hardware components:

| 5V Power Supply | → | ESP32 Microcontroller | → | RTC Module (DS3231) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ↓ |  |  |  |  |
| Servo Motor (Dispenser) | → | Motor Driver (L298N) | → | LCD/OLED Display |  | Buzzer (Alert) |
|  |  | ↓ |  |  |  |  |
|  |  | WiFi / IoT Connectivity | → | Cloud (Blynk/Firebase) | → | Caregiver Notification |

Figure 4.1: Block Diagram of Automated Medical Dispenser using ESP32

4.2 Circuit Diagram Description

The circuit consists of the following key connections:

- ESP32 GPIO pins are connected to the RTC module via I2C bus (SDA and SCL pins).

- The servo motor is connected to a designated ESP32 PWM-capable GPIO pin through the motor driver.

- The LCD/OLED display is connected via I2C or SPI interface.

- The buzzer is connected to an ESP32 GPIO pin through a transistor driver circuit.

- Push buttons are connected to ESP32 input pins with pull-up resistors for schedule configuration.

- All components are powered through a regulated 5V supply from the main 12V adapter.

4.3 System Flowchart

The system operation follows this logical sequence:

START SYSTEM

Initialize RTC & Servo Motor

Read Current Time from RTC

Current Time = Scheduled Time?

Trigger Motor Pin HIGH (Dispense)

Wait 1000ms

Return Motor Pin LOW (Reset)

Wait 60 seconds

Loop back to Read Current Time

Figure 4.2: System Operation Flowchart

4.4 System Working Principle

Upon power-on, the ESP32 initializes all peripherals including the RTC module, servo motor, LCD display, and buzzer. The system continuously reads the current time from the DS3231 RTC module and compares it to the pre-programmed medication schedule stored in flash memory. When a match is found, the ESP32 sends a PWM signal to the servo motor via the motor driver, rotating the pill container to align the correct compartment with the dispensing outlet.

Simultaneously, the buzzer activates to alert the patient, and the LCD/OLED display updates to show the medication name and dosage information. After a brief dispense cycle of approximately one second, the motor resets to its home position. If IoT connectivity is active, the event is logged to the cloud platform and a push notification is sent to the caregiver's device.

CHAPTER 5: OBJECTIVES

The project aims to achieve the following specific objectives:

- Design and implement an automated medicine dispensing system using ESP32 as the central microcontroller.

- Ensure accurate medication timing using the DS3231 RTC module for reliable time-based scheduling.

- Provide audible and visual alerts using buzzer and LCD/OLED display when medicines are due.

- Enable remote monitoring and caregiver notifications using IoT platforms (Blynk / Firebase).

- Reduce medication errors through automation, eliminating reliance on human memory.

- Improve patient adherence and overall safety for elderly and chronically ill patients.

- Develop a user-friendly interface for medication schedule setting using push buttons.

- Create a cost-effective and portable healthcare solution accessible to a broad population.

- Support caregivers with real-time notifications and remote status monitoring.

CHAPTER 6: ADVANTAGES

The Automated Medical Dispenser offers several significant advantages over manual medication management:

- Ensures accurate and timely medicine dispensing based on pre-programmed schedules.

- Reduces the chances of missed doses due to forgetfulness or busy schedules.

- Minimizes human errors in medication intake, especially critical for multi-drug regimens.

- Provides automatic alerts and reminders via buzzer and display notifications.

- Enables remote monitoring through IoT connectivity, giving caregivers peace of mind.

- Improves patient safety and medication adherence, reducing hospitalizations.

- Particularly useful for elderly patients, chronically ill individuals, and post-surgery care.

- Cost-effective and portable solution that is affordable compared to commercial dispensers.

- Easy to operate with a user-friendly interface requiring minimal technical knowledge.

CHAPTER 7: APPLICATIONS

The Automated Medical Dispenser has wide-ranging applications across multiple healthcare settings:

- Elderly care homes and senior citizen residences where medication management is a daily challenge.

- Home healthcare monitoring for patients managing chronic conditions independently.

- Hospitals and clinics for outpatient medication dispensing and patient education.

- Chronic disease management for conditions including diabetes, hypertension, and cardiac diseases.

- Assisted living centers where staff-to-patient ratios make individual monitoring difficult.

- Rehabilitation centers for post-treatment medication compliance monitoring.

- Post-surgery patient care where precise dosing schedules are medically critical.

- Remote patient monitoring systems integrated with telemedicine platforms.

- Rural healthcare services where access to regular medical check-ups is limited.

- Smart healthcare IoT solutions as part of connected hospital infrastructure.

CHAPTER 8: RESULTS

The prototype of the Automated Medical Dispenser was assembled and tested in a laboratory environment. The following results were observed during functional testing:

| TC # | Test Description | Expected Result | Actual Result | Status |
| --- | --- | --- | --- | --- |
| TC-01 | Scheduled time match | Motor rotates, pill dispensed | Motor rotated correctly | PASS |
| TC-02 | Buzzer alert on dispense | Buzzer sounds for 2 seconds | Buzzer activated | PASS |
| TC-03 | LCD display update | Time and dose info shown | Display updated correctly | PASS |
| TC-04 | Power cycle recovery | RTC retains correct time | Time preserved after reset | PASS |
| TC-05 | IoT notification | Caregiver notified via app | Notification received | PASS |
| TC-06 | Manual button override | Schedule setting works | Settings saved correctly | PASS |
| TC-07 | Multiple dose schedule | Both doses dispensed | Correct dispensing at both times | PASS |
| TC-08 | Wrong time input | No dispensing action | No unintended activation | PASS |

All test cases passed successfully. The system demonstrated reliable medication scheduling with accurate time-based dispensing, effective alert mechanisms, and stable IoT connectivity throughout the testing period.

CHAPTER 9: FUTURE SCOPE

Several enhancements have been identified for future iterations of the Automated Medical Dispenser:

AI Integration

Integration with AI for dosage analysis and patient behavior monitoring, enabling adaptive scheduling based on patient response and health data patterns.

Mobile Application

A dedicated mobile app for prescription upload, dosage tracking, and historical medication adherence reports accessible to both patients and healthcare providers.

Cloud Storage

Cloud data storage for long-term medication history, enabling healthcare professionals to monitor patient compliance over extended periods.

Voice Assistant Integration

Voice assistant integration with Amazon Alexa or Google Assistant for hands-free reminders and status queries.

Camera-Based Verification

Camera module integration to visually confirm medicine intake, providing objective compliance data to caregivers.

Multi-Channel Alerts

SMS, Email, and WhatsApp alert system for comprehensive caregiver notification across multiple communication channels.

Biometric Authentication

Biometric authentication using fingerprint or face recognition to prevent unauthorized access to the dispenser.

Automatic Refill Notification

Pill level sensing to trigger automatic refill notifications when medication stock falls below a threshold.

Battery Backup

Battery backup system with power failure alerts to ensure continued operation during power outages.

Multi-User Support

Family medication tracking with support for multiple users and individual medication profiles on a single device.

CHAPTER 10: CONCLUSION

The Automated Medical Dispenser using ESP32 provides a reliable and cost-effective solution for timely and accurate medicine dispensing. The system successfully addresses the critical problem of medication non-adherence through intelligent automation, removing the dependency on human memory for routine medication management.

The RTC module (DS3231) ensures precise scheduling accuracy, maintaining correct time even through power interruptions. The servo motor mechanism provides reliable and repeatable dispensing action. The buzzer and LCD/OLED display combination delivers clear and timely patient notifications. The IoT connectivity via ESP32's WiFi enables seamless caregiver monitoring from remote locations.

All functional test cases were successfully completed, validating the system's core capabilities. The prototype demonstrates that with commodity hardware at minimal cost, a functional and reliable automated medication management solution can be built and deployed in real-world healthcare settings.

The modular architecture of the system provides an excellent foundation for future enhancements including AI integration, mobile applications, biometric authentication, and integration with hospital management systems, making this project a valuable contribution to the growing field of IoT-enabled smart healthcare solutions.

CHAPTER 11: PROJECT OUTCOMES

The following outcomes were successfully achieved upon completion of this project:

- Successfully designed and implemented a complete automated medicine dispensing system using ESP32 as the central control unit.

- Achieved accurate medication scheduling using the DS3231 RTC module with sub-minute timing precision.

- Ensured reliable pill dispensing through a servo motor mechanism with consistent rotational control.

- Implemented a comprehensive alert system combining buzzer audio notifications and LCD/OLED display output for effective patient notification.

- Established stable IoT connectivity for remote monitoring and real-time caregiver notifications via cloud platforms.

- Validated system reliability through complete functional testing with all test cases passing successfully.

- Demonstrated a scalable and cost-effective design applicable to home use, elderly care facilities, and clinical environments.

CHAPTER 12: TEAM MEMBERS DETAILS

The following team members contributed to the design, implementation, and documentation of this project:

| S.No | Member Name | Contribution | Student ID |
| --- | --- | --- | --- |
| 1 | Devineni Prem Nag | Hardware Design & Circuit Implementation | 2300049018 |
| 2 | D. Shanmukha Venkata Sai | Firmware Development & Programming | 2300049019 |
| 3 | M. Hemanth Kumar | IoT Integration & Cloud Platform Setup | 2300049034 |
| 4 | SK. Sajeeth Shareef | Testing, Documentation & Report Writing | 2300049047 |

REFERENCES

Reference Books

[1] Kolban, N. (2017). Getting Started with ESP32. Neil Kolban. Available at https://leanpub.com/kolban-ESP32

[2] Bahga, A. & Madisetti, V. (2014). Internet of Things: A Hands-On Approach. Universities Press.

[3] Blum, J. (2013). Exploring Arduino. John Wiley & Sons.

[4] Valvano, J. W. (2014). Embedded Systems: Introduction to ARM Cortex-M Microcontrollers. Self-published.

[5] Wilmshurst, T. (2006). Designing Embedded Systems with PIC Microcontrollers. Newnes.

Websites and Web Links

[6] Espressif Systems – https://www.espressif.com

[7] Arduino – https://www.arduino.cc

[8] IEEE Xplore – https://ieeexplore.ieee.org

[9] Random Nerd Tutorials – https://randomnerdtutorials.com

[10] Blynk IoT Platform – https://blynk.io

[11] Firebase by Google – https://firebase.google.com

[12] ThingSpeak IoT Platform – https://thingspeak.com